Solar PV Detection Model: Comprehensive Training and Validation Report

This report summarizes the training process, performance, and detailed validation characteristics of the YOLOv8 object detection model developed for the Solar Rooftop PV Detection and Area Estimation Pipeline.

- Model and Training Summary

| Parameter | Detail | Source |
| --- | --- | --- |
| Model Architecture | YOLOv8s | - |
| Model Size | 72 layers, 11,12,745 parameters, GFLOPs | Validation_metrics.txt |
| Training Epochs | 50 | - |
| Input Image Size | 640 pixels | - |
| Best Performance Epoch | Epoch 31 | Results.csv |

- Overall Final Performance Metrics

| Metric | Overall Value | Source |
| --- | --- | --- |
| Mean Average Precision (mAP50-95) | 0.561 | Validation_metrics.txt |
| Mean Average Precision (mAP50) | 0.815 | Validation_metrics.txt, BoxPR_curve.png |
| Mean Precision | 0.816 | Validation_metrics.txt |
| Mean Recall | 0.771 | Validation_metrics.txt |
| Optimal F1 Score | 0.79 | BoxF1_curve.png |

- Training Progress Analysis

The learning curves demonstrate a successful training process:

- Losses: The training losses (train/box_loss, train/cls_loss, train/dfl_loss) exhibit a rapid and consistent decline over 50 epochs, indicating effective learning.

- Validation Stability: Validation losses stabilize after approximately 10-15 epochs, confirming convergence and avoiding significant overfitting.

- Metric Improvement: The primary metrics (metrics/mAP50(B), metrics/mAP50-95(B)) show a steep rise and then level off, sustaining high performance throughout the later epochs.

- Detailed Per-Class Performance

The model uses three distinct classes: 0, SolarPanel, and Solarpanel

Per-Class Quantitative Metrics

| Class | Instances | Precision | Recall | AP50 | Map50-95 | F1 Peak |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 780 | 0.921 | 0.872 | 0.931 | 0.690 | ~0.90 |
| SolarPanel | 271 | 0.752 | 0.642 | 0.699 | 0.453 | <0.70 |
| Solarpanel | 2063 | 0.774 | 0.798 | 0.815 | 0.540 | ~0.80 |

Confusion Matrix Analysis (Normalized)

The normalized confusion matrix (true classes vs. predicted outcomes) highlights classification strengths and weaknesses:

| True Class | Proportion Correctly Predicted (Recall) | Primary Error Mode (Missed Detections - FN) | False Positives from Background |
| --- | --- | --- | --- |
| 0 | 0.89 | 11% missed as background | 12% of all '0' predictions were background |
| SolarPanel | 0.70 | 30% missed as background | 12% of all 'SolarPanel' predictions were background |
| Solarpanel | 0.83 | 17% missed as background | 75% of all 'Solarpanel' predictions were background |

- Performance Observations and Recommendations

- High-Confidence Performance (Class 0): The model performs exceptionally well on Class 0 (likely representing standard rooftop PV installations).

- Weakest Class: The SolarPanel class is the weakest performer, with the lowest mAP@0.5 (0.699) and a high miss rate (30%). This suggests the data for this specific class label may be insufficient, poorly annotated, or represents highly complex features.

- False Positive Issue: The Solarpanel class, despite good recall, suffers from a high rate of False Positives, where 75% of the model's predictions for this class were actually background. This indicates the model struggles to differentiate this panel type from non-panel features.